EXPT 5

clc;

clear; close;

count=csvRead('C:\scilab projects\user defined functions\count.dat');

for i= 1:3

scf();

mu(i)= mean(count(:,i));

sigma(i)=stdev(count(:,i));

histplot(24,count(:,i));

xtitle('histogram of traffic intersection' + string(i));

end

MeanTotal=mean(mean(count));

disp('mean for individual traffic intersection is=',mu);

disp('stanard deviation for individual traffic intersection is=',sigma); disp('overall mean =',MeanTotal);

EXPT 6

clc;

clear;

close;

exec('C:\scilab_projects\user defined functions\pskmod.sci');

exec('C:\scilab_projects\user defined functions\qammod.sci');

N = 4; // number of bits to be grouped

M = 2^N;

x = 0:M-1;

z = pskmod(x, M);

scf(); // create a new figure

subplot(1,2,1);

scatter(real(z), imag(z),"fill");

h = gca(); // get current axes

h.data_bounds = [-1.5,1.5,-1.5,1.5];//adjust axes boundries as per need

xtitle('E3204M-ary PSK', 'In-phase', 'Quadrature');

y = qammod(x, M);

subplot(1,2,2);

scatter(real(y), imag(y),"fill");

h = gca();

h.data_bounds = [-3.5,3.5,-3.5,3.5];

Expt 7

Code :

clc;

clear;

close;

exec('C:\scilab_projects\user defined functions\awgn.sci');

data_bits = 100000;

b = rand(1, data_bits) > 0.5;

s = 2 * b - 1;

SNRdB = 0:9;

for k = 1:length(SNRdB)

y = awgn(s, SNRdB(k));

error = 0;

for c = 1:data_bits

if ((y(c) > 0 & s(c) == -1) | (y(c) < 0 & s(c) == 1)) then

error = error + 1;

end

end

BER(k) = error / data_bits;

end

SNR = 10.^(SNRdB / 10);

BER_thBPSK = (1 / 2) * erfc(sqrt(SNR));

BER_thQPSK = erfc(sqrt(SNR));

scf();

semilogy(SNRdB, BER, 'r', 'LineWidth', 2);

xtitle('E3204 Probability of Error Graph', 'SNR (dB)', 'BER');

xgrid;

semilogy(SNRdB, BER_thBPSK, 'k', 'LineWidth', 2);

semilogy(SNRdB, BER_thQPSK, 'b', 'LineWidth', 2);

legend('Simulated BPSK', 'Theoretical BPSK', 'Theoretical QPSK');

Expt 8

clc;

clear all;

p_xy=input('Enter joint probability matrix ');

[m,n]=size(p_xy);

for i=1:m

px(i)=0;

for j=1:n

px(i)=px(i)+p_xy(i,j);

end

end

disp('*E3204*P(X) matrix is:',px);

for i=1:n

py(i)=0;

for j=1:m

py(i)=py(i)+p_xy(j,i);

end

end

disp('P(Y) matrix is:',py);

hx=sum(-px.*log2(px));

disp('Input entropy H(X) is:',hx);

hy=sum(-py.*log2(py));

disp('Output entropy H(Y) is:',hy);

h_xy=sum(-p_xy.*log2(p_xy));

disp('Joint Entropy H(X,Y) is:',h_xy);

hxby=h_xy-hy;

hybx=h_xy-hx;

disp('Conditional Entropy H(X/Y) is:',hxby);

disp('Conditional Entropy H(Y/X) is:',hybx);

ixy=hx+hy-h_xy;

disp('Mutual Information I(X,Y) is:',ixy);

EXpt 9

clc;

clear;

n=input("(E3204)Enter the codeword length in LBC(n):");

k=input("Enter the number of msg bit in LBC(k):");

p=input("Enter the parity matrix");

g=[eye(k,k)p];

disp("generator matrix",g);

d=[0 0 0;0 0 1;0 1 0;0 1 1;1 0 0;1 0 1;1 1 0;1 1 1]

c=modulo(d*g,2);

disp("All codeword",c);

Hw=zeros(2^k,1);

for i=1:2^k

wt=0;

for j=1:n

if c(i,j)==1 then

wt=wt+1;

end

end

Hw(i)=wt;

end

y=[c,Hw];

disp('code vector with hamming weight',y);

dmin=Hw(2);

for i=2:2^k

if dmin>Hw(i) then

dmin=Hw(i);

end

end

disp('Minimum hamming distance is',dmin);

td=dmin-1;

disp('Error detection capabilityh id code(in bits)is:',td);

tc=(dmin-1)/2;

disp('error correction capability of code(in bits)is',tc);

disp('Transpose of parity matrix is',p);

H=[p',eye(n-k,n-k)];

disp(' parity check matrix is',H);

disp('Transpose of parity check matrix is',H');

r=input("enter the received codeword");

synd=modulo(r*H',2);

disp('Synd for received codeword is:',synd);

for i=1:size(H',1)

if H'(i,1:n-k)==synd then

r(i)=1-r(i);

disp('Error is at location',i);

end

end

disp('corrected codeword is',r);

EXPT10

clc;

clear;

exec('C:\scilab_projects\user defined functions\deconv.sci');

n = input('E3204Enter the length of codeword : ');

k = input('Enter the length of message : ');

gen_coff = input('Enter the generator coefficient : ');

m = input('Enter the message : ');

y2 = [1];

a = zeros(1, n - k);

z1 = [y2 a];

x = conv(z1, m);

x1 = pmodulo(x, 2);

[q, r] = deconv(x1,gen_coff);

r1 = [zeros(1, length(x1) - length(r)),r];

codeword = bitxor(x1,r1); //codeword = (x1 & ~r1) | (~x1 & r1);

disp('CYCLIC CODEWORD= ',codeword*1);

rec = input('Enter the received codeword : ');

[q,r] = deconv(rec, gen_coff);

syn = pmodulo(r, 2);

disp('Syndrome for received codeword is:',syn);

if syn == 0 then

disp('No error in received code, No need of correction ');

else

disp('Non zero syndrome indicates Error in received code');

syn = [zeros(1, length(x1) - length(syn)),syn];

y2 = zeros(n, n);EXPT NO.10 Roll.no:E3204

e = eye(n, n);

for i = 1:n

[q, r] = deconv(e(i,:),gen_coff);disp('Conditional Entropy H(X/Y) is:',hxby);

disp('Conditional Entropy H(Y/X) is:',hybx);

ixy=hx+hy-h_xy;

disp('Mutual Information I(X,Y) is:',ixy);

r1 = [zeros(1,n - length(r)),r];

y2(i,:) = pmodulo(r1, 2);

end

for i = 1:n

if syn == y2(i,:) then

break

end

end

correctedCode=bitxor(rec,e(i,:));

disp('CORRECTED CODEWORD= ',correctedCode*1);

end